AX5400 Wi-Fi6 Access Point

BRAX EA1305

IPv4/v6

PoE

Wi-Fi6

TR069

Firewall

QoS

Overview

BRAX EA1305 is a AX5400 Wi-Fi6 Ceiling Mount access point, which can be deployed in various scenarios such as enterprises, hotels, and shopping malls to provide users with high-speed WLAN access. In addition to basic routing, Wi-Fi, and Firewall function, it can also support remote management, providing convenient methods for unified deployment and management.

Hardware Specification

| Platform | CPU: Qualcomm Dual-Core A53 1.0GHz Flash: 128MB, RAM: 512MB |
| --- | --- |
| Standards | 2.4GHz IEEE 802.11 b/g/n/ax 2T2R 5GHz IEEE 802.11a/n/ac/ax 4T4R |
| Data rate | 2.4GHz: 574Mbps (2ss 40MHz) 5GHz: 4804Mbps (4ss 160MHz) |
| Antennas | Internal Omni 2.4GHz 2*2 Internal Omni 5GHz 4*4 Gain: 5dBi |
| Interface | 1×GE WAN port, support POE 2×GE LAN port 1×DC power port |
| Button | 1×Reset |
| LED | 1×System LED |
| Power consumption | <18W |
| Power supply | 802.3at PoE 12 V/1.5A DC |
| Physical specification | Dimension: 235*180*43mm |
| Environment | Work temperature: 0°C ~ +50°C Storage temperature: -40°C ~ +70°C Humidity: 5% ~ 95% |

| Software Specification |  |
| --- | --- |
| Wireless LAN - Support 802.11 a/b/g/n/ac/ax Standards - 20/40/80/160 MHz bandwidth - Channel adaption - MSSID + Guest network - WPS: PBC/PIN modes - WPA/WPA2/WPA3 security Network Protocols and Features - Operation modes: Bridge or Routed - Connection modes: PPPoE/DHCP/Static - IPv4, IPv6, IPv4/IPv6 - DHCP v4/v6 server - Static (v4/v6) routing - NAT/ALG/DMZ/virtual server - ARP/ICMP/DNS proxy - NTP - VLAN - VPN | Management - WebUI local management - TR069 remote management - Universal Plug and Play (UPnP) - Online and local software upgrade - Local diagnostic Firewall - Built-in firewall - Defense of DoS, ARP attacks/Port scan - MAC/Port filtering - 802.1x Quality of Service Control - 802.11e WMM - Classification of service flow - Port/VLAN/ IP rate limitation |

BRAX EA1305

AX5400 Wi-Fi6 Access Point

Application Scenario